ESC – IoT Solutions

ARIS – Thread demo

Kristóf Féja

Revision 1.0, 05.02.2018

Quick Start Guide

©2018 by ARROW

All rights reserved. No part of this manual shall be reproduced, stored in a retrieval system, or transmitted by any means, electronic, mechanical, photocopying, desktop publishing, recording, or otherwise, without written permission from the publisher. No patent liability is assumed with respect to the use of the information contained herein. While every precaution has been taken in the preparation of this document, the publisher and author assume no responsibility for errors or omissions. Neither is any liability assumed for damages resulting from the use of the information contained herein. All terms mentioned in this manual that are known to be trademarks or service marks are listed below. In addition, terms suspected of being trademarks or service marks have been appropriately capitalized. ARROW cannot attest to the accuracy of this information. Use of a term in this document should not be regarded as affecting the validity of any trademark or service mark.

# Revision History

| Revision, Date | Editor | Subject (major changes) |
| --- | --- | --- |
| Revision 1.0, 05.02.2018 | Kristóf Féja | First release |

# Table of Contents

Revision History	3

Table of Contents	4

Engineering Solutions Center	5

How to run a Thread demo with the ARIS kits	5

Introduction	5

Hardware Requirements	5

ARIS GATEWAY programming	5

ARIS EDGE/ARIS EDGE S3 programming	6

Operating the demo	6

# Engineering Solutions Center

The mission of Arrow’s Engineering Solutions Center is to support the field team in their design activities ranging from NPI proposals, consultancy in complex areas like software, IoT, FPGAs, high-end to complete system concepts and Arrow’s ready-to-use solutions.

Through the TestDrive board loan program, the ESC provides many supplier development boards and Arrow developed solutions to enable quick design starts. Design, customization, prototyping and certification services are available from Arrow’s comprehensive 3rd Party Network.

# How to run a Thread demo with the ARIS kits

## Introduction

Following steps will help You to create your own Thread demo using the ARIS GATEWAY and ARIS EDGE/ARIS EDGE S3 devices.

In this setup the ARIS GATEWAY board will act as the Leader on the Thread network and the ARIS EDGE or ARIS EDGE S3 devices as Router Eligible End Devices (REED).

## Hardware Requirements

The following equipment is needed to flash and run the demo:

- 1 ARIS GATEWAY board

- Minimum 2 ARIS EDGE or ARIS EDGE S3 boards

- Segger J-Link (minimum EDU)

- J-Link 9-Pin Cortex-M Adapter

## ARIS GATEWAY programming

- Use the provided USB cable to connect the board to your laptop.

- Open _jlink_ folder

- Double-click on program_s7.bat

This will flash the software onto the S7 MCU on the board.

The next step is to flash the MGM111 multiprotocol module.

- Connect your J-Link to your laptop and connect the 9-Pin Adapter to your J-Link device.

- Connect the 9-Pin Adapter to J16 header and remove jumper J18.

- Double-click on program_msm_s7.bat then select the serial number of your J-Link device. (You can find it on its bottom.)

- When done programming, disconnect the J-Link from the board and insert J18 jumper back.

- Disconnect the board and put it aside. Now it is time to program the ARIS EDGE / ARIS EDGE S3 devices.

## ARIS EDGE/ARIS EDGE S3 programming

- Use the provided USB cable to connect the board to your laptop.

- Double-click on program_s1.bat

This will flash the software onto the S1 MCU on the board.

The next step is to flash the MGM111 multiprotocol module.

- Connect your J-Link to your laptop and connect the 9-Pin Adapter to your J-Link device.

- Connect the 9-Pin Adapter to J4 header and remove jumper J14.

- Double-click on program_msm_s1.bat then select the serial number of your J-Link device. (You can find it on its bottom.)

- When done programming, disconnect the J-Link from the board and insert J14 jumper back.

- Repeat above steps on your other boards.

## Operating the demo

The ARIS GATEWAY will create the network automatically when powered.

There is a 300 seconds join window during which end devices can join to the network. The blue LED is on while the join window is active.

Power up your ARIS EDGE/ARIS EDGE S3 devices and press SW3 on each to join the network. When joined and sending data the blue LED will blink.

By pressing SW2 on the ARIS GATEWAY a serial port over USB is enabled for showing status and collected data.

Pressing SW1 on the ARIS GATEWAY will reset the network state. Following this action node association has to be done again.

https://www.arrow.com